Response to the Reviewer 2

Reviewer 2’s comment 1:

I'm satisfied with the response of the authors. Therefore, I'm glad to recommend this work for publication in Photonics Research.

Reply— We thank the reviewer for the acknowledgement of our manuscript and recommendation for publication.

Response to the Reviewer 3

Reviewer 3’s comment 1:

The cyclic transformation of a set of orthogonal OAM modes is an essential building block for universal high-dimensional information processing. This work makes an essential step to extend unitary transformations to more modes. And the manuscript is well written. The author has also addressed the questions raised by the previous two reviewers very well. I would like to recommend the publication of this manuscript.

Reply—The reviewer has precisely summarizes our work. We sincerely thank the reviewer for their positive comments and recommendation for publication.

Reviewer 3’s comment 2:

However, I have two questions for improving the manuscript.

1. The averaged efficiency of cyclic transformation of six OAM modes is about 96%. What factors limit this efficiency? Can it be further improved and how? Will it affect the fidelity of single photon quantum Pauli-X gate?

Reply—We thank the reviewer for his/her valuable feedbacks which have helped us to significantly improve our manuscript. We hope that the reviewer is satisfied with our responses and the revised manuscript.

The average efficiency depends mainly on the transmittance and reflectance of the FP cavity to the OAM modes. To ensure high transmittance for the resonant OAM modes and high reflectance for the off-resonant OAM modes, the target resonant OAM mode should perfectly match the cavity mode with no degeneracy, and is well separated from the off-resonant modes in the frequency domain.

Experimentally, in order to increase the transmission of the FP1 cavity in the l=±3,  p=0 modes, we reduced the beam waist of the incident light beam to half of that of the cavity mode (w0=50 μm) and observed a significant enhancement of transmission from ~20% to ~70% in the l=±3 modes behind the FP1 cavity. If the Q-plates can be replaced by SLMs that realize amplitude-phase modulation, the OAM modes will always remain at high purity as the azimuthal mode indices l increase. It is reasonable to believe that the transmittance of the FP1 cavity will also be improved in this mode. In turn, the average efficiency will also be increased.

In addition, since our FP cavities are flat-convex cavities. As a result, the transverse modes of the vortex beam from the convex side of the FP2 cavity are poor relative to those from the planar side.  This deformation of the transverse modes also leads to a reduction in the collection efficiency of the single-mode fiber, which in turn reduces the average efficiency. The improvement is made by discarding the scheme of one flat-convex etalon and instead building a normal FP cavity utilizing two flat-concave mirrors and a piece of piezoelectric ceramic attached to one of the two separated mirrors.

We believe that the factors affecting the fidelity of the high-dimensional single-photon quantum Pauli-X gate are mainly the transmittance and reflectance of FP cavity to the OAM modes, the accuracy of the MZI-controlled phases, and the efficiency of projective measurements on the OAM modes and their superpositions. The average efficiency is the one that will affect the fidelity of single-photon Pauli-X gate.

To respond to this point, we have added the paragraphs “The average efficiency depends mainly on…… which is also reflected in our experimental results [36]” above Fig. 5 in page 8 and 9 for our revised manuscript in order to discuss the factors that limit the efficiency, and ways to improve it. We have also added the sentences “To summarize, the factors affecting the fidelity of the high-dimensional single-photon quantum Pauli-X gate…… Thus, the average efficiency is the one that will affect the fidelity of the gate.” in the last two paragraph in page 9 in order to discuss the effect of the efficiency to the fidelity of the high-dimensional single-photon quantum Pauli-X gate.

Reviewer 3’s comment 3:

2. The authors reduce the beam waist of incident light beam to half of that of cavity mode in order to increase transmission of FP1 cavity in the 𝑙 =± 3, 𝑝 = 0 modes. Is this waist spot size optimal? Will the transmission of approximately 75% affect the efficiency of cyclic transformation?

Reply— This waist spot size is indeed optimal to enable the highest purity for l=±3,  p=0 modes, see Ref. [34] in the main text. We also experimentally verified this result. Further decreases of the waist of the OAM modes do not contribute to the transmittance of the FP cavity for the higher-order OAM modes.

Next, we analyze in detail the effect of transmission and reflection of the FP cavities for the l=3 OAM mode to the efficiency of the cyclic transformation. After OAM mode with l=2 enters the cyclic transformation section, it goes through the second QWP-QP-QWP combination to shift to l=3 mode in the experiment. The transmittance of FP1 cavity to l=3 mode is about 70%. The reflected mode of l=3 propagates in the left arm of the MZI and reaches the planar side of the FP2 cavity. Most of this OAM mode is transmitted through FP2. Only a small portion of the mode (about 9% of the total energy content of light beam with l=3 mode) is reflected by the FP2 cavity and exits the left arm of the MZI after reflected by the PBS2. The OAM mode is changed to l=3, which is not contained in the encoded azimuthal mode index of {l=-3,  -2,  -1, 0, 1, 2}, and is not collected in the single-mode fiber.

On the other hand, the transmitted mode of l=3 travels along the right arm of the MZI and finally exits after reflected by the PBS2. As a result, it changes to the mode with l=-3. The lower transmission of the FP cavity leads to a lower conversion efficiency of l=2 mode to l=-3 mode. We conclude that a lower transmittance of the FP cavity will affect the efficiency of the cyclic transformation. However, as discussed above, the transmittance of the FP cavity can be further enhanced by replacing the Q-plates to SLMs. We believe that the efficiency of the cyclic transformation can be further improved.

To respond to this point, we have changed the sentences to“As suggested by Ref. [34]……  higher-order l mode is the highest”, and “The measured data imply……do not enhance the transmittance of the FP cavities” in the paragraph under Eq. (7) in section 4 to reason that the waist spot is optimal. We have also added the sentences “The average efficiency depends mainly on…… will be further improved in this mode” at the second last paragraph in page 8 for our revised manuscript in order to show the effect of the transmittance to the average efficiency.